Module 6.2 – Answer to Exercise 3

A valid test would be "real performance in simulated situation", e.g. measuring a voltage drop between two points.  One item would be sufficient in a detailed observation sheet/scoring key is provided because it is very unlikely that someone who has not learned the task, perform it correctly by chance.

Instruction for trainees:

You have in front of you a multimeter, a booklet describing the instrument (manufacturer's reference), 2 cords with plugs and 2 measuring points marked A and B.

Measure a voltage between the points A and B the way you have learned to in this course.  You will be considered qualified if you make no more than 2 minor faults.

Instructions for instructors:

Give written instructions to the trainees, together with the equipment listed in the instructions.

Observe one trainee at a time, and check his performance using the observation sheet provided.  Cross the appropriate column for each subtask.

Score performance using the scoring key shown in the observation sheet.

continued.../

SCORING KEY / OBSERVATION SHEET

|  | YES | NO | SCORE |
| --- | --- | --- | --- |
| 1. Place multimeter horizontally |  |  | 5 |
| 2. Check needle position and adjust |  |  | 5 |
| 3. Set selection switch to "direct current", "alternating current" |  |  | 14 |
| 4. Choose the appropriate range |  |  | 14 |
| 5. Connect the cords to inlets |  |  | 12 |
| 6. Connect the cords to the measurement points |  |  | 12 |
| 7. Adapt the range if necessary |  |  | 14 |
| 8. Read and interpret the value |  |  | 14 |
| 9. Disconnect the instrument |  |  | 5 |
| 10. Check by another multimeter if the result is not plausible |  |  | 5 |

PASS = 90 points or more

FAIL = less than 90