ITU/TDG Form 4

| JOB: Machine Operator (aaa) |  |  | Station: BBB |  |  |  | Completed by: CCC |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TASK: Drill pieces |  |  |  |  |  |  | Date: DDD |
| Initial conditions and triggering event | No | Subtasks |  | F | I | Perform. Diff. | S/K/A Requirements |
| Work order. Drilling machine in | 1 | Put work piece in jig correctly |  | VF | VI |  | 1.1 Identify work pieces and jib |
| order. Pieces in box to be drilled |  |  |  |  |  |  | 1.2 Discriminate b/w small and big screw |
|  |  |  |  |  |  |  | 1.3 Fasten screws in correct sequence |
| Location, Tools, Refs. | 2 | Hold jig firmly on drilling platform |  | VF | I |  | 2.1 Identify position of work pieces under drilling |
| Workshop. Drilling machine, job | 3 | Start the machine |  | VF | I |  | 3.1 Identify start-stop control |
| pieces, safety regulations and | 4 | Oil on tip of the drill |  | VF | VI |  | 4.1 Identify tip of drill |
| equipment |  |  |  |  |  |  | 4.2 Discriminate b/w too much/little/adequate oil |
|  |  |  |  |  |  |  | on tip of drill |
| Terminating event | 5 | Lower down the drill on work |  | VF | I | ease up | 5.1 Identify handle-bar |
| Drilled pieces placed correctly in the |  | piece by steady pressure |  |  |  | when almost | 5.2 Apply adequate pressure (motor skill) |
| box | 6 | When hole is through, pull off |  | VF | I | through = BE | 5.3/6.1 Identify effects on drill bit of lowering/lifting |
| Standards |  | the drill from work piece |  |  |  | CAREFUL | the handle-bar |
| Hole drilled according to process des | 7 | Take out the work piece from jig |  | VF | I |  | 7.1 Loosen screws |
| cribed under subtasks. Piece placed | 8 | Stock the work piece in correct |  | VF | I |  | 8.1 Identify correct position of work pieces in box |
| correctly in box. Safety precautionsd |  | place/way in box |  |  |  |  |  |
| according to regulations. 100%! |  |  |  |  |  |  |  |